Fretted Synth Audio

Piezo Drum Pad to MIDI ©2009

frettedsynth@att.net

www.frettedsynth.com

PDP2MIDI VST plugin drum pad to MIDI converter

PDP2MIDI is designed to be used with a piezo type drum pad plugged directly into your soundcard. PDP2MIDI turns the audio input into a MIDI signal usable by any drum VST\I.  Five different versions are included! a two channel version, a seven channel version and an eleven channel version for MIDI out. CompleteDrums which provides SoftDrumLTD as a multi wave player and SingleDrum provides a single channel trigger\wave player. Each Version except SingleDrum also contains a group channel to switch between three different note numbers using a continuous controller pedal set to CC#16, useful  as a hihat controller and more.

Note: not all hosts can work with this type of MIDI VST, as it is an audio in, MIDI out plugin. However using a VST chainer plugin like energyXT or other should make it useful in most hosts or use CompleteDrums\SingleDrum.

Have also tested using a guitar for the trigger input with very good results. You should be able to trigger with almost any audio source provided that the breaks between the peaks drop below the threshold. Does not work too well with piezo type acoustic drum triggers but I have got good results from these on a kick drum. SingleDrum has also been tested as a drum replacer with good results.

Manual

NOTE TO MULTI CORE PROCESSOR USERS - There is a problem with running multiple instances of PDP2MIDI on multiple CPU machines, when using a host that supports more than one processor. This will be fixed with SynthEdit v1.1,  I will then be able to fix this bug.

There is a work-around for now - Copy and paste the .dll file and rename (example) PDP2MIDI.dll copy\paste and rename PDP2MIDI2.dll, PDP2MIDI3.dll  ect... You may make as many copies as you need instances just give different names to all of them and load them separate. Sorry about the bug will fix as soon as SE v1.1 is released.

NOTE: PDP2MIDI, PDP2MIDI_7 and PDP2MIDI_11 all can be ran together without having to rename. You only have to rename if you need more than one instance of a single version.

Controls

All controls move by a linear (up, down) mouse movement, except the buttons and the dropdown controls you left click. You may fine tune a control by holding your computer keyboards Control  <Ctrl> key while moving a control.

On\Off - Turns the gate on or off for that input channel, also reduces CPU use when a channel is switched off.

Threshold - Sets the threshold where the gate will trigger, Turn down for low input signals, turn up for a high input signal.

Velocity - Adjusts the velocity tracking for different input levels to setup a MIDI output of  1-127 note velocity tracking.

Curve - Allows you to select from a Linear, exponential, inverted exponential 1or inverted exponential 2 velocity curves. Also has an option to switch the velocity tracking off.

Note # - Selects the MIDI output note for that channel, from note 36 - C2 to note 60 - C4.

Closed Note # - Selects the MIDI output note when the Open\Close control is held down, from note 36 - C2 to note 60 - C4.

Foot Note # - Selects the MIDI output note from the action of the open\close trigger, from note 36 - C2 to note 60 - C4.

Open\Close - Used to switch the group from open to closed position, also has a foot closed trigger, Default MIDI CC# 16 may also be mapped through most hosts.

Foot Velocity - Sets the velocity value for the foot closed trigger.

MIDI Channel - Global setting for the MIDI transmit channel.

Setup

- 1. Start with the velocity tracking OFF it make it much more easy to hear false triggers. Use the velocity curve control to set velocity tracking to off.

- 2. Set the MIDI note you want to trigger.

- 3. Begin by setting the “Threshold” control. Play your lightest hit keep increasing the threshold until your lightest hit wont trigger anymore, then turn the “Threshold” down just a little so your lightest hit triggers again.

- 4. Turn the velocity tracking back on, use the velocity curve control to set velocity curve type. Use the velocity control to adjust so your hardest hit will yield a MIDI velocity level of 127.

A note about cross-trigger: When more then one drum pad is mounted on the same stand cross-triggering can occur, Cross-triggering is when one drum on a stand is hit and another pad on the same stand triggers from the vibration.

You can check for cross-triggering by watching the LED for each channel, If other pads trigger (led lights) other then the hit\triggered pads LED.

If you notice a cross-triggered channel turn its “Threshold” up until the cross-triggering stops, The down side is you lose some of your light trigger ability but with pads on the same stand sometimes it cant be helped? Sometimes its helps to move the drums mounting position on the stand, kind of changes the way the vibration travels.

Its also useful to check the setup with the velocity OFF sometimes cross-triggers and false triggers are of a very low velocity so you don’t hear them..

Group\Hi Hat controller - The Open\Close control is designed to simulate a hi-hat being opened and closed, its default MIDI controller is CC# 16 may also be mapped through most hosts.

When your external MIDI controller is open the hi-hat is open, When the external MIDI control is closed the hi-hat is closed. There is also a third note triggered by the external continuous control being closed. Designed to simulate a foot closed hi-hat, its MIDI velocity level is adjustable from the plugins UI and is a fixed value.

Continuous Controller Map \ CC#’s

16 - Open-Close control

All other controls must be mapped through your host.

PDP2MIDI License

The Software is provided as-is, without warranty of any kind. Fretted Synth

(the Author) disclaims all warranties relating to the Software,

whether express or implied, including but not limited to any implied

warranties of merchantability and fitness for a particular purpose,

and all such warranties are expressly and specifically disclaimed.

The Author shall not be liable for any indirect, consequential or

incidental damages arising out of the use or inability to use

the Software even if the Author has been advised of the possibility

of such damages or claims. The user of the Software bears all risk

as to the quality and performance of the Software.

The Software is Copyright ©2009 Fretted Synth Audio. The Software is not

public domain, and is protected by the copyright laws.  You may not sell,

sublicense, distribute, rent, lease, modify, translate, convert to another

programming language, decompile, or disassemble the Software for any

purpose.

Thanks to God

Thanks to Jeff McClintock for SynthEdit

For the graphic tools knobman and skinman Thanks to g200kg

For custom 3rd party SynthEdit modules used in PDP2MIDI,  Thanks to:

Chris Kerry and David Haupt

Fretted Synth Audio

frettedsynth@att.net

www.frettedsynth.com

VST is a trademark of Steinberg Soft- and Hardware GmbH